# Meridian Retrieval Appliance - Service Manual

Applies to appliance models MRA-4200, MRA-4400 and MRA-8800, firmware 7.3 and later. Document revision D-2291.

# 1. Diagnostic Error Codes

The appliance reports faults as an alphanumeric code on the front panel display. Codes persist in the event log for 90 days.

## 1.1 Index Subsystem

ERR_5521 - Vector index checksum mismatch after an unclean shutdown.

Corrective action: Rebuild the index with 'mra-admin index --rebuild'. Expect 40 minutes per million vectors.

ERR_5522 - Index shard unreachable; the shard process exited without flushing.

Corrective action: Restart the shard. If ERR_5522 recurs within one hour, replace part number PN-8891-A.

ERR_5530 - Embedding dimension mismatch between the index and the configured model.

Corrective action: The index was built at a different dimensionality. Re-embed the corpus; do not edit the config.

## 1.2 Query Subsystem

ERR_6104 - Query latency exceeded the configured p99 budget of 800 ms.

Corrective action: Lower efSearch from 256 to 128 and re-measure. Recall will fall by roughly two points.

ERR_6110 - Reranker model failed to load.

Corrective action: Verify the model cache at /var/lib/mra/models. Free at least 4 GB.

# 2. Field-Replaceable Parts

Order parts by the exact part number. Substitutions void the warranty.

| Part Number | Description | Models | Lead Time |
| --- | --- | --- | --- |
| PN-8891-A | Index shard controller board | MRA-4400, MRA-8800 | 3 days |
| PN-8891-B | Index shard controller board, revision B | MRA-8800 only | 10 days |
| PN-7712-C | Front panel display assembly | all models | 1 day |
| PN-9004-F | Cooling fan module, 80 mm | MRA-4200, MRA-4400 | same day |

# 3. Maintenance Schedule

## 3.1 Quarterly

Verify index recall against the stored ground-truth query set. A drop of more than three points indicates corpus drift, not hardware fault.

## 3.2 Annually

Replace PN-9004-F regardless of apparent condition. Bearing failure is not detectable before it occurs.